# Smart Emergency Vehicle Alert System Using ESP32

# Abstract

The Smart Emergency Vehicle Alert System is designed to detect ambulance, police, and fire engine sirens using two MAX9814 microphone modules connected to an ESP32. The controller compares the sound received by the left and right microphones to identify the direction of the approaching emergency vehicle. The system displays the direction on an OLED display and uses LED indicators to alert the driver. This prototype demonstrates a low-cost embedded solution for improving road safety and helping drivers respond quickly to emergency vehicles.

# Objectives

Detect emergency vehicle sirens.

Determine the direction of the emergency vehicle.

Display direction on an OLED display.

Provide visual alerts using LEDs.

Improve road safety using an embedded system.

# Hardware Components

| Component | Purpose |
| --- | --- |
| ESP32 Development Board | Main controller |
| MAX9814 Microphone Module (2) | Detect siren sound |
| 0.96-inch OLED Display (I2C) | Display alerts and direction |
| Red LED | Emergency indicator |
| Green LED | Normal status indicator |
| 220 Ω Resistors | LED current limiting |
| Breadboard | Prototype assembly |
| Jumper Wires | Connections |
| USB Cable / 5V Supply | Power supply |

# Software Used

Arduino IDE

ESP32 Board Package

Adafruit SSD1306 Library

Adafruit GFX Library

# Working Principle

Two microphones continuously monitor surrounding sounds. The ESP32 reads both microphone signals and compares their sound levels. When the sound exceeds the preset threshold, the controller recognizes a possible emergency siren. By comparing the left and right microphone values, the ESP32 determines whether the emergency vehicle is approaching from the left, right, or front. The OLED display shows the detected direction while the LEDs indicate system status.

# Block Diagram

# Advantages

Low-cost prototype

Simple hardware design

Real-time siren detection

Easy to install and maintain

Improves driver awareness

Low power consumption

# Applications

Cars and taxis

Buses

Public transport

Smart transportation systems

Traffic safety research

Educational embedded projects

# Future Scope

AI/FFT-based siren recognition

Automatic music volume control

GPS integration

Distance estimation

Vehicle-to-Vehicle (V2V) communication

Mobile application integration

# Conclusion

The Smart Emergency Vehicle Alert System is a simple and effective embedded solution for detecting emergency vehicle sirens and informing drivers about the direction of approaching emergency vehicles. By combining dual microphones, an ESP32, OLED display, and LED indicators, the system enhances road safety and can be expanded into a full automotive assistance system.